TRAINER PLC OMRON CP2E N30DRA

DAN HMI Weintek MT8072IP

# Gambar Bagian dan Spesifikasi

- IP Address Komponen

  - IP Address PLC	: 169.254.84.210

  - IP Address HMI	: 169.254.84.200

  - IP Address PC		: Sesuaikan dengan 169.254.84.xxx

- Gambar Bagian dan Spesifikasi

  - Gambar Bagian dan Spesifikasi

- Spesifikasi

  - PLC Omron CP2E N30DRA

  - HMI Weintek MT8072IP

  - Socket Power 220VAC

  - MCB 1 Phase 10A

  - Saklar On/Off

  - Fuse 5A/10A

  - Button Emergency NO dan NC

  - Selektor 3 mode atau 2 kontak NO

  - Button NO hijau

  - Button NC Merah

  - Lampu Indikator Merah 220VAC

  - Lampu Indikator Kuning 220VAC

  - Lampu Indikator Hijau 220VAC

  - Buzzer 220VAC

  - Banana Jack Female 22mm merah

  - Banana Jack Female 22mm hitam

  - Banana Jack Male Merah

  - Banana Jack Male Hitam

  - Socket RJ45 Male to Female

  - Socket RS232 Male to Female

  - Socket USB Male to Female

- Gambar Wiring

- Gambar Komunikasi

  - PC to PLC

  - PLC to HMI

# Tutorial Cx-Programmer

- Buka Software Cx-Programmer

- Klik New

- Setting device type

- Klik Ok dan buat program seperti dibawah ini

- Tuliskan Alamat Input dan Outputnya.

- coba program dengan work online

- lalu transfer kedalam PLC

- Lalu masuk kedalam Run Mode/Monitoring Mode

# Tutorial Easy Builder Pro

- Buka Software Easybuilder Pro

- klik New

- pilih HMI yang sesuai

- lalu tekan OK

- pilih New Device/Server

- pilih tipe device

- sesuaikan dengan PLC yang digunakan

- pilih komunikasi ethernet FINS/TCP

- setting IP sesuai dengan IP PLC yang digunakan

- lalu OK

- pilih window mana yang mau digunakan

- masuk ke menu object

- pilih set bit

- drag ke tengah

- atur pada general seperti gambar berikut

- lalu apply dan OK

- ulangi untuk tombol selanjutnya

- pilih bit lamp untuk menambahkan lampu indikator

- atur seperti gambar berikut

- lalu apply dan OK

- pergi ke menu project

- pilih Download PC > HMI

- cari IP/HMI Name

- klik download

- lalu klik online simulation

- HMI dapat digunakan

# Tutorial Komunikasi HMI - PLC dengan Ethernet

OMRON CJ/CS/CP (Ethernet - FINS/TCP)

Supported Series: OMRON CJ Series, CS Series, CP Series +Ethernet Module. (Ethernet FINS)

Website: http://www.omron.com/

*On initialization, switch from RUN MODE to MONITOR MODE.

HMI Setting:

| Parameters | Recommended | Options | Notes |
| --- | --- | --- | --- |
| PLC type | OMRON CJ/CS/CP (Ethernet - FINS/TCP) |  |  |
| PLC I/F | Ethernet |  |  |
| Port no. | 9600 |  |  |
| PLC sta. no. | 0 |  |  |

PLC Setting:

| Communication mode | Ethernet (FINS/TCP) protocol |
| --- | --- |

Import Tags:

- Export the *.cxr file in cx-programmer -> variables

[Important]: Only addresses with tag name can be imported, and address type is required

- Open EasyBuilder Pro, and set the drive parameters in [System Parameter Settings], click [Import Tags].

- Select *.cxr file

- Check the TAG that the project needs to use.

- Imported tag information successfully.

Device Address:

| Bit/Word | Device type | Format | Range | Memo |
| --- | --- | --- | --- | --- |
| B | CIO_Bit | DDDDDdd | 0 ~ 3276715 | Channel I/O (CIO) |
| B | W_Bit | DDDDDdd | 0 ~ 3276715 | Work Area (WR) |
| B | H_Bit | DDDDDdd | 0 ~ 3276715 | Holding Area (HR) |
| B | A_Bit | DDDDDdd | 0 ~ 3276715 | Auxiliary Relay (AR) |
| B | D_Bit | DDDDDdd | 0 ~ 3276715 | Data Memory (DM) |
| B | T_Bit | DDDDDdd | 0 ~ 3276715 | Timer (TIM) |
| B | C_Bit | DDDDDdd | 0 ~ 3276715 | Counter (CNT) |
| B | C_Flag | DDDD | 0 ~ 4095 |  |
| B | T_Flag | DDDD | 0 ~ 4095 |  |
| B | EM0_Bit ~ EM18_Bit | DDDDDdd | 0 ~ 3276715 | Extend Memory |
| W | CIO | DDDDD | 0 ~ 32767 | Channel I/O (CIO) |
| W | W | DDDDD | 0 ~ 32767 | Work Area (WR) |
| W | H | DDDDD | 0 ~ 32767 | Holding Area (HR) |
| W | A | DDDDD | 0 ~ 32767 | Auxiliary Relay (AR) |
| W | C | DDDDD | 0 ~ 32767 | Counter (CNT) |
| W | T | DDDDD | 0 ~ 32767 | Timer (TIM) |
| W | D | DDDDD | 0 ~ 32767 | Data Memory (DM) |
| W | EM0 ~ EM18 | DDDDD | 0 ~ 32767 | Extend Memory |

Wiring Diagram:

Diagram 1

Ethernet cable:

# Tutorial Komunikasi HMI TO PLC dengan RS232

OMRON CJ/CS/CP

Supported Series: OMRON CP1E, CP2E, CP1L, CP1H, CJ1M, CJ2M, CJ1H, CJM1G, CS1H and CS1G. (Host Link Protocol FINS command), this driver supports Extend Addressing Mode.

Website: http://www.omron.com/

HMI Setting:

| Parameters | Recommended | Options | Notes |
| --- | --- | --- | --- |
| PLC type | OMRON CJ/CS/CP |  |  |
| PLC I/F | RS232 | RS232, RS422, RS485 |  |
| Baud rate | 9600 | 9600~115200 |  |
| Data bits | 7 | 7 or 8 |  |
| Parity | Even | Even, Odd, None |  |
| Stop bits | 2 | 1 or 2 |  |
| PLC sta. no. | 0 | 0-31 | Host Link Station No. |
| Protocol | Multi-read | Multi-read / Single read |  |

| Online simulator | YES | Extend | address | YES |
| --- | --- | --- | --- | --- |
| Broadcast | NO |  |  |  |
| Communication | Host Link Protocol |

*Support communications between HMI and PLC in pass-through mode *Set LW-9903 to 2 to enhance the speed of download/upload PLC program in pass-through mode PLC Setting: mode

Device Address:

| Bit/Word | Device type | Format | Range | Memo |
| --- | --- | --- | --- | --- |
| B | CIO_Bit | DDDDDdd | 0 ~ 3276715 | Channel I/O (CIO) |
| B | W_Bit | DDDDDdd | 0 ~ 3276715 | Work Area (WR) |
| B | H_Bit | DDDDDdd | 0 ~ 3276715 | Holding Area (HR) |
| B | D_Bit | DDDDDdd | 0 ~ 3276715 | Data Memory (DM) |
| B | A_Bit | DDDDDdd | 0 ~ 3276715 | Auxiliary Relay (AR) |
| B | T_Bit | DDDDDdd | 0 ~ 3276715 | Timer (TIM) |
| B | C_Bit | DDDDDdd | 0 ~ 3276715 | Counter (CNT) |
| Bit/Word | Device type | Format | Range | Memo |
| B | C_flag | DDDD | 0 ~ 4095 |  |
| B | T_flag | DDDD | 0 ~ 4095 |  |
| B | LR_Bit | DDDdd | 0 ~ 19915 |  |
| B | EM0_Bit ~ EM18_Bit | DDDDDdd | 0 ~3276715 | Extend Memory |
| W | T | DDDDD | 0 ~ 32767 | Timer (TIM) |
| W | H | DDDDD | 0 ~ 32767 | Holding Area (HR) |
| W | D | DDDDD | 0 ~ 32767 | Data Memory (DM) |
| W | A | DDDDD | 0 ~ 32767 | Auxiliary Relay (AR) |
| W | W | DDDDD | 0 ~ 32767 | Work Area (WR) |
| W | C | DDDDD | 0 ~ 32767 | Counter (CNT) |
| W | CIO | DDDDD | 0 ~ 32767 | Channel I/O (CIO) |
| W | EM0 ~ EM18 | DDDDD | 0 ~ 32767 | Extend Memory |
| W | LR | DDD | 0 ~ 199 |  |

Import Tags:

- Export the *.cxr file in cx-programmer -> variables

[Important]: Only addresses with tag name can be imported, and address type is required

- Open EasyBuilder Pro, and set the drive parameters in [System Parameter Settings], click [Import Tags].

- Select *.cxr file

- Check the TAG that the project needs to use.

- Imported tag information successfully.

Wiring Diagram:

# Diagram 1

RS-232

The serial port pin assignments may vary between HMI models, please click the following link for more information.

HMI

PLC

k

Lin

RS232 9P

D

-

Sub Male

Rx

S

2

D

Tx

3

RD

GND

9

GND

4

RS

5

CS

# Diagram 2

RS-485 4W (CP1H/CP1L CP1W-CIF11 RS485 4W : 9P D-Sub to Termainals)

The serial port pin assignments may vary between HMI models, please click the following link for more information.

| HMI |  | PLC |
| --- | --- | --- |
| Link |  | CP1W-CIF11 RS485 4W Terminal |
| Rx- |  | SDA |
| RX+ |  | SDB |
| Tx- |  | RDA |
| Tx+ |  | RDB |
| GND |  | FG |

CP1W-CIF11: SW1 ON, others OFF.